| Project Title: | A Real-Time Haptic Rendering System Integrating Physics-Based Simulation and OpenGL Visualisation |  | Submission Date: | 1/05/2026 |  |
| --- | --- | --- | --- | --- | --- |
| Student Name: | 11349510 |  |  |  |  |

| Project Risk | Severity |  |  | Potential |  |  | Score (Severity x Potential) | Mitigation Measures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L | M | H | L | M | H | L=1, M=2, H=3 |  |
| Electrical faults during wiring/testing (short circuit, incorrect connections) | X |  |  |  | X |  | 2 | Use low-voltage systems; power off before modifying wiring; check connections before powering; use insulated connectors |
| Injury from moving mechanical parts (rotating joints, links) |  | X |  |  | X |  | 4 | Limit actuator torque; avoid contact during operation; test at low power; implement watchdog shutdown |
| Actuator overheating during prolonged operation |  | X |  | X |  |  | 2 | Monitor device temperature; limit runtime; allow cooling periods; avoid sustained high torque |
| Mechanical failure (belt slip, loose components) |  | X |  |  | X |  | 4 | Secure all components; inspect before testing; avoid excessive loading; operate within tested limits |
| Control instability causing unexpected motion or oscillations |  |  | X |  | X |  | 6 | Use conservative control gains; include damping; implement torque limits and rate limiting; test incrementally |
| Communication failure (loss of command stream) |  | X |  |  | X |  | 4 | Implement watchdog timeout to zero torque; validate communication before operation |
| Software bugs causing incorrect behaviour |  | X |  |  | X |  | 4 | Incremental testing; logging and debugging; isolate subsystems during development |
| Component failure (e.g. actuator failure during project) |  | X |  |  | X |  | 4 | Design for partial operation; maintain flexibility in testing; prioritise core functionality |
| Use of tools during assembly (cuts, slips) | X |  |  |  | X |  | 2 | Use appropriate tools; follow workshop safety procedures; maintain tidy workspace |
| Extended computer use (eye strain, fatigue) | X |  |  |  |  | X | 3 | Take regular breaks; maintain ergonomic posture; adjust screen setup |